# The quarter car model

The quarter car model of a vehicle is used to describe the vertical dynamic of a wheel and its suspension.

Blocks m1 and m2 represent, respectively, the car and wheel masses involved in the suspension system, while springs k1 and k2 are the car and wheel masses stiffness (also called spring constants).

The quantities x1 and x2 represent, respectively, the displacements of the car and wheel masses (assuming that only a vertical motion is allowed), and the vertical direction is conventionally considered positive; xin is the displacement input acting on the wheel, modelling road profile.

The damper c is modelling the damping action of the suspension system which, strictly speaking, is only consisting of a spring (k1, in this case) and the damper itself. In case of a passive system, the value of c is a constant parameter set by the manufacturer and cannot be modified. In case of semi-active systems, c can be modified while driving.

The dynamic equation describing the suspension system is the following:

|  | (1) |
| --- | --- |

where x1, x2, x1 and x2 represent, respectively, the masses’ velocities and accelerations.

The previous equation can be split into two separate equations, yielding the vertical accelerations for the two masses:

x1=k1x2-k1x1-cx1+cx21m1                       (2)

x2=k2xin+cx1-cx2+k1x1-k1+k2x21m2            (3)

Typical values for parameters k1, k2, m1 and m2 are the following:

| Parameter | Value |
| --- | --- |
| Car body mass (m1) | 380 Kg |
| Wheel body mass (m2) | 31 Kg |
| Suspension stiffness (k1) | 29.000 N/m |
| Wheel stiffness (k2) | 228.000 N/m |
| Damping (c) | 1.500 Ns/m |

# The Skyhook controller

To improve the performance of a vehicle and to guarantee a better comfort of the vehicle occupants, a semi-active suspension system is adopted where a control low is used to change the Damping c in relation to the suspended mass and wheel speed.

The Skyhook principle is the following:

x1-x2x1≥0 ⟹c=6000Nsm

x1-x2x1<0 ⟹c=1500Nsm